�����������������������������������������������������������������ͻ

�                                                                 �

�                           README for                            �

�                                                                 �

�    UM85C418 VL BUS IDE DRIVER & WINDOWS IDE DRIVER, VER 1.0     �

�                                                                 �

�����������������������������������������������������������������ͼ

(1)

UM82C418 VL BUS IDE DRIVER & WINDOWS IDE DRIVER are saved in

this floppy.There are 3 files in this floppy disk as followed :

1. README

2. IDEDRV.SYS

3. IDEDRV.386

where IDEDRV.SYS is the DOS ide driver,

and IDEDRV.386 is the WINDOWS ide driver.

(2).

����������������������������������������������������Ŀ

�    DOS Driver for UM82C418 VL-Bus IDE Controller   �

������������������������������������������������������

If you want to install DOS ide driver, please follow the following

rule.

i. edit the file config.sys at the root directory as the following .

device=c:\idedrv.sys           (we assume the root was c:\)

ii. Reboot or reset your pc.

(3).

��������������������������������������������������������Ŀ

� Microsoft Windows version 3.1 disk driver for UM82C418 �

����������������������������������������������������������

The file IDEDRV.386 is the Microsoft Windows 3.1 virtual device driver

for UM82C418 disk ide controller.

1. Why you need IDEDRV.386 ?

Microsoft Windows version 3.1 supports 32-bit disk access, but the

disk data transfer between host and the IDE adapter is still limited

by the 16-bit ISA bus. So If you want to speed up your hard disk drives

access under Windows through the 32-bit VL-Bus, you should install this

driver .

2. How to install IDEDRV.386 ?

If you want to install IDEDRV.386 for Windows , please follow the

the following steps .

i. Copy the driver IDEDRV.386 into your system in the appropriate path.

You had better to copy yhe driver IDEDRV.386 into the Windows

SYSTEM directory.

Ex. copy a:\idedrv.386 c:\windows\system

(we assume idedrv.386 was in a: , and windows package was in c:\)

ii. Check the file SYSTEM.INI , and make sure the following lines exist in the

the [386Enh] section as below

[386Enh]

32BitDiskAccess=ON

device=*int13

If the statements do not exist, add them into the [386Enh] section.

If the statements do exist, but the 32BitDiskAccess is set to "OFF",

yoy should change it to "ON".

iii. Install the virtual device driver "IDEDRV.386".

Add the following command line into the [386Enh] section of the

SYSTEM.INI file:

device=[drive:][\path\]IDEDRV.386

where the [drive:] and [\path\] point to the directory that contains

IDEDRV.386 file.

Ex. device=c:\windows\system\idedrv.386

(we assume you copy the file idedrv.386 into c:\windows\system)

*NOTE: setting idedrv.386 must specify the full path .

iv . Delete the "device=*wdctrl" device setting.

Delete the command line "device=*wdctrl" from the [386Enh] section

of your SYSTEM.INI file, if it exists.

v. Restart Windows , and it will bring up your PC with the new system

configuration.